| ## Part Name | CTCXXXX | CTDXXXX | CTFXXXX | CTGXXXX-S | CTGXXXX-F |
| --- | --- | --- | --- | --- | --- |
| Out line |  |  |  |  |  |
| Rated Current | ±(3A ~ 50A) |  |  | ±(50 ~ 600A) |  |
| Measure Range | ±150A |  |  | ±900A |  |
| Dimension(W*H*D) | 33*23*13 | 16*23*17 |  | 40*28*28 |  |
| Supply voltage | ±15V(±5%) |  |  |  |  |
| Current consume | <±20mA |  |  |  |  |
| V0 offset(0 A) | <0±40mV |  |  |  |  |
| Vo (at I rated) | V0±4V |  |  |  |  |
| Vo linearity | <1% |  |  |  |  |
| Thermal drift | <2mV/℃ |  |  |  |  |
| Output internal R | <100ohms |  |  |  |  |
| Load resistance | 10Kohms(suggest) |  |  |  |  |
| AC isolation | Min. 2.5KV |  |  |  |  |
| Isolated Resis. | >500Mohms(at 500Vdc) |  |  |  |  |
| Response time | <3uS(at 90% of Irated*3) |  |  |  |  |
| Operate Temp. | -10℃ ~ +80℃ |  |  |  |  |
| Storage Temp. | -25℃ ~ +85℃ |  |  |  |  |
| Working Freq. | 50KHz |  |  |  |  |
| Remarks | Open loop |  |  |  |  |
| ## Part Name | CTGXXXX-B | CTHXXXX | CTIXXXX | CTLXXXX | CTMXXXX |
| Out line |  |  |  |  |  |
| Rated Current | ±(50A ~ 600A) | ±(3A ~ 50A) |  |  |  |
| Measure Range | ±900A | ±150A |  |  | -50A,+150A |
| Dimension(W*H*D) | 40*28*28 | 51*21*19 | 34*21*19 | 17*21*17 |  |
| Supply voltage | ±15V(±5%) |  |  |  | +15V(±5%) |
| Current consume | <±20mA |  |  |  | <+20mA |
| V0 offset(0 A) | <0±40mV |  |  |  | <2500±50mV |
| Vo (at I rated) | V0±4V |  |  |  | V0±2V |
| Output linearity | <1% |  |  |  |  |
| Thermal drift | <2mV/℃ |  |  |  |  |
| Output internal R | <100ohms |  |  |  |  |
| Load resistance | 10Kohms(suggest) |  |  |  |  |
| AC isolation | Min. 2.5KV |  |  |  |  |
| Isolated Resis. | >500Mohms(at 500Vdc) |  |  |  |  |
| Response time | <3uS(at 90% of Irated*3) |  |  |  | <5uS(at Irated) |
| Operate Temp. | -10℃ ~ +80℃ | -10℃ ~ +85℃ |  |  |  |
| Storage Temp. | -25℃ ~ +85℃ |  |  |  |  |
| Working Freq. | 50KHz |  |  |  |  |
| Remarks | Open loop |  |  |  |  |